# INSTALLING NEW CHUNKS O’ FRUTI GRAPHICS

Surface Preparation All surfaces must be thoroughly cleaned before applying your new Chunks O’ Fruti graphics. Follow the instructions below for specific material cleaning procedures. Plastics and Glass Thoroughly wipe surface clean with a window cleaner. Remove any dried matter such as tape or glue with a single edge razor blade. Painted Metals and Base Metals These surfaces may be cleaned with any household cleaner such as Fantastic or 409. Dry the surface with a lint free cloth before the solvent evaporates. If you have just repainted the surface should dry for at least 72 hours prior to application of the graphics. Temperature Requirements The temperature of the room and the temperature of the intended surface should be between 45 and 90 degrees Fahrenheit for best results. Very high humidity (above 90 percent) may affect the application process. Materials Needed Small cutting knife or razor blade. (an Exacto Knife is ideal for the job) A Spray bottle filled with a mixture of dish soap and water. Please mix one teaspoon of Dove or other NON-DETERGENT soap with one gallon of water. Plastic Squeegee and a sponge Dry Towel 2" Roll of Masking Tape

| 1. Using small tabs of masking tape, tack the graphic to the intended surface. This is to insure proper placement before the liner is removed. |  |
| --- | --- |
| 2. Apply a single horizontal strip of 2" masking tape to the top of the graphic. The strip of should be split between the graphic and the intended surface with 1" on each side (as shown). This strip will be used as a hinge that will insure the graphic does not move during the application process. |  |
| 3. Hold the bottom edge of the hinged graphic away from the intended surface and remove the liner to reveal the sticky backing of the vinyl. Save this piece of liner for use later in the application process. |  |
| 4. To assist in proper placement, spray the intended surface with the mixture of soap and water. (This coating of water reduces the tackiness of the vinyl, letting you lift and re-place the graphic if necessary. |  |
| 5. While still holding the hinged graphic away from the intended surface. Using the squeegee and starting from the top, begin to marry the graphic to the intended surface. |  |
| Suggestions: - Work from top to bottom. - Do not let the entire graphic fall onto the intended surface. - If lines or creases develop, lift and reposition the graphic.. |
| 6. Once the graphic is in proper position and lying perfectly flat on the intended surface, you can now start forcing the water out from beneath the graphic. Starting from the center working outward, use firm strokes with the squeegee or sponge to force the water out from beneath the graphic. |  |
| 7. |  |
| Squeegee out any remaining water bubbles using the piece of left over liner. Lay the piece of left over liner over the graphic, with shiny side of liner facing you. Proceed to apply hard pressure to squeegee out any remaining water or air bubbles. |  |
| Installing graphics is a simple follow-the-steps process. Although it takes very little time, it is not something to rush. With a little concentration and patience you can easily install the graphics.. |  |